EXPERIMENT 1.2

IDENTIFICATION AND ANALYSIS OF UNDER-MODULATION IN AM

1. AIM

To identify and analyze the under-modulation condition of an amplitude-modulated (AM) signal by calculating the modulation index and verifying that m is less than 1 using MATLAB.

2. APPARATUS / SOFTWARE REQUIRED

Computer or laptop

MATLAB software

MATLAB Editor

3. THEORY AND FORMULA

In amplitude modulation, the amplitude of a high-frequency carrier changes according to the modulating signal. The modulation index indicates the extent of carrier amplitude variation.

m = Am / Ac

Here, Am is the peak amplitude of the modulating signal and Ac is the peak amplitude of the carrier. Under-modulation occurs when m < 1. At m = 1, the signal is 100% modulated; when m > 1, over-modulation occurs.

Given: Am = 2, Ac = 5

m = 2 / 5 = 0.4 = 40%

Since 0.4 is less than 1, the AM signal is under-modulated. Its envelope remains above zero for the selected positive carrier amplitude and can be followed by an envelope detector.

4. MATLAB CODE

clc;

clear;

close all;

% Signal parameters

Ac = 5;          % Carrier amplitude

Am = 2;          % Modulating signal amplitude

fc = 100;        % Carrier frequency in Hz

fm = 5;          % Modulating frequency in Hz

fs = 10000;      % Sampling frequency in Hz

% Calculate modulation index

m = Am / Ac;

fprintf('Modulation Index = %.2f\n', m);

fprintf('Modulation Percentage = %.2f%%\n', m*100);

% Check modulation condition

if m < 1

disp('Condition: Under-modulation');

elseif m == 1

disp('Condition: 100 percent modulation');

else

disp('Condition: Over-modulation');

end

% Time vector

t = 0:1/fs:2/fm;

% Generate signals

message = Am*cos(2*pi*fm*t);

carrier = Ac*cos(2*pi*fc*t);

am_signal = Ac*(1 + m*cos(2*pi*fm*t)) ...

.* cos(2*pi*fc*t);

% Calculate AM envelope

envelope = Ac*(1 + m*cos(2*pi*fm*t));

% Plot signals

figure;

subplot(3,1,1);

plot(t, message, 'b', 'LineWidth', 1.2);

title('Modulating Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,1,2);

plot(t, carrier, 'k');

title('Carrier Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(3,1,3);

plot(t, am_signal, 'r');

hold on;

plot(t, envelope, 'b--', 'LineWidth', 1.2);

plot(t, -envelope, 'b--', 'LineWidth', 1.2);

title('AM Signal - Under-Modulation');

xlabel('Time (s)');

ylabel('Amplitude');

legend('AM Signal', 'Upper Envelope', 'Lower Envelope');

grid on;

5. MATLAB OUTPUT

The output contains the modulating signal, carrier signal, and AM waveform with upper and lower envelopes.

6. RESULT

The modulation index was calculated as m = Am/Ac = 2/5 = 0.4 (40%). Since m < 1, the signal satisfies the under-modulation condition. The modulating signal, carrier signal, and AM waveform with its envelope were plotted successfully in MATLAB.