EXPERIMENT 4: AM SUPERHETERODYNE / IF SIGNAL ANALYSIS

# 1. AIM

To generate an AM signal, mix it with a local oscillator to obtain an intermediate frequency (IF) signal, and recover the original message signal using envelope detection in MATLAB.

# 2. APPARATUS / SOFTWARE REQUIRED

Computer or laptop

MATLAB software

MATLAB Editor for entering and running the program

# 3. THEORY

In this experiment, an AM signal is generated using a message signal and a carrier signal. The AM signal is then multiplied with a local oscillator signal to perform frequency mixing. An IF filter is used to obtain the intermediate-frequency component. The recovered signal is then obtained using envelope detection based on the analytic signal.

Given parameters:

Message frequency, fm = 500 Hz

Carrier frequency, fc = 10000 Hz

Local oscillator frequency, fLO = 14550 Hz

Intermediate frequency, IF = 14550 − 10000 = 4550 Hz

# 4. MATLAB CODE

clc;
clear;
close all;

% Parameters
fs = 100000;
t = 0:1/fs:0.02;
fm = 500; % Message frequency
fc = 10000; % Carrier frequency
fLO = 14550; % Local Oscillator (IF = 4550 Hz)

% Message Signal
m = cos(2*pi*fm*t);

% Carrier Signal
c = cos(2*pi*fc*t);

% AM Signal
am = (1 + m).*cos(2*pi*fc*t);

% Local Oscillator
LO = cos(2*pi*fLO*t);

% Frequency Mixing
mixed = am .* LO;

% IF Filter
[b,a] = butter(4,5000/(fs/2));
IF = filter(b,a,mixed);

% Envelope Detection
env = abs(hilbert(IF));

% Display
figure;
subplot(4,1,1)
plot(t,m)
title('Message Signal')
grid on

figure;
subplot(5,1,2)
plot(t,c)
title('Carrier Signal')
grid on

subplot(5,1,3)
plot(t,am)
title('AM Signal')
grid on

subplot(5,1,4)
plot(t,IF)
title('Intermediate Frequency (IF) Signal')
grid on

subplot(5,1,5)
plot(t,env)
title('Recovered Signal')
grid on

# 5. MATLAB OUTPUT

The output shows the message signal, carrier signal, AM signal, intermediate-frequency (IF) signal, and recovered signal.

# 6. RESULT

The AM signal was generated successfully and mixed with the local oscillator. The intermediate-frequency signal was obtained using the IF filter, and the message was successfully recovered using envelope detection.