EXPERIMENT 5: FREQUENCY MODULATION (FM) AND DEMODULATION

# 1. AIM

To generate a frequency modulated (FM) signal, determine its instantaneous frequency and modulation index, and recover the original message signal using FM demodulation in MATLAB.

# 2. APPARATUS / SOFTWARE REQUIRED

Computer or laptop

MATLAB software

MATLAB Editor for entering and running the program

# 3. THEORY

In frequency modulation, the frequency of the carrier signal varies according to the instantaneous amplitude of the message signal, while the carrier amplitude remains constant.

FM modulation index: β = fd / fm

where fd is the maximum frequency deviation and fm is the message frequency.

For this experiment, fm = 50 Hz and fd = 100 Hz. Therefore, β = 100/50 = 2. The carrier frequency is fc = 500 Hz.

The instantaneous frequency is given by: f_inst = fc + fd cos(2πfmt).

# 4. MATLAB CODE

clc;
clear;
close all;

% Parameters
Am = 1;             % Message signal amplitude
Ac = 1;             % Carrier signal amplitude
fm = 50;            % Message frequency (Hz)
fc = 500;           % Carrier frequency (Hz)
Fs = 20000;         % Sampling frequency (Hz)
T = 0.1;            % Duration (seconds)
t = 0:1/Fs:T-1/Fs;

% Frequency deviation
fd = 100;            % Maximum frequency deviation (Hz)

% Modulation index
beta = fd/fm;

% Message signal
m = Am*cos(2*pi*fm*t);

% FM signal generation
s = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t));

% Instantaneous frequency
f_inst = fc + fd*cos(2*pi*fm*t);

% FM Demodulation using phase differentiation
analytic_signal = hilbert(s);
instant_phase = unwrap(angle(analytic_signal));
inst_freq = [diff(instant_phase)*Fs/(2*pi), 0];

% Recover message signal
m_rec = (inst_freq - fc)/fd;

% Remove small numerical errors using low-pass filtering
[b,a] = butter(4, 2*fm/(Fs/2));
m_rec = filtfilt(b,a,m_rec);

% Normalize recovered signal
m_rec = m_rec - mean(m_rec);
m_rec = m_rec/max(abs(m_rec));

% Display modulation index
fprintf('Message Frequency = %d Hz\n', fm);
fprintf('Carrier Frequency = %d Hz\n', fc);
fprintf('Frequency Deviation = %d Hz\n', fd);
fprintf('FM Modulation Index = %.2f\n', beta);

% Plot message signal
figure;
subplot(4,1,1);
plot(t,m,'b','LineWidth',1.2);
grid on;
title('Original Message Signal');
xlabel('Time (s)');
ylabel('Amplitude');

% Plot carrier signal
subplot(4,1,2);
c = Ac*cos(2*pi*fc*t);
plot(t,c,'r','LineWidth',1);
grid on;
title('Unmodulated Carrier Signal');
xlabel('Time (s)');
ylabel('Amplitude');

% Plot FM signal
subplot(4,1,3);
plot(t,s,'k','LineWidth',1);
grid on;
title('Frequency Modulated (FM) Signal');
xlabel('Time (s)');
ylabel('Amplitude');

% Plot recovered message signal
subplot(4,1,4);
plot(t,m,'b','LineWidth',1.2);
hold on;
plot(t,m_rec,'r--','LineWidth',1.2);
grid on;
title('FM Demodulation: Original vs Recovered Message');
xlabel('Time (s)');
ylabel('Amplitude');
legend('Original Message','Recovered Message');

% Plot instantaneous frequency
figure;
plot(t,f_inst,'m','LineWidth',1.2);
grid on;
title('Instantaneous Frequency of FM Signal');
xlabel('Time (s)');
ylabel('Frequency (Hz)');

# 5. MATLAB OUTPUT

The first output shows the original message signal, unmodulated carrier signal, frequency-modulated signal, and comparison of the original and recovered message. The second output shows the instantaneous frequency of the FM signal.

# 6. RESULT

The FM signal was generated successfully in MATLAB. The modulation index was calculated as β = fd/fm = 100/50 = 2. The instantaneous frequency varied between 400 Hz and 600 Hz, and the original message signal was successfully recovered through FM demodulation.