EXPERIMENT: AM MODULATION INDEX

1. AIM

To calculate the modulation index of an amplitude-modulated (AM) signal using the carrier and modulating signal amplitudes in MATLAB, and to plot the baseband, carrier, AM signal with its envelope, and demodulated output.

2. APPARATUS / SOFTWARE REQUIRED

Computer or laptop

MATLAB software

MATLAB Editor for entering and running the program

3. THEORY AND FORMULA

In amplitude modulation, the amplitude of a high-frequency carrier varies according to the instantaneous value of the low-frequency modulating signal.

Modulation index:  m = Am / Ac

Where Am is the peak amplitude of the modulating signal and Ac is the peak amplitude of the carrier signal. For this experiment, Am = 2 and Ac = 5.

m = 2 / 5 = 0.4

Modulation percentage = m x 100 = 40%. Since m is less than 1, the signal is under-modulated.

4. MATLAB CODE

clc;

clear;

close all;

% Signal parameters

Ac = 5;

Am = 2;

fc = 100;

fm = 5;

fs = 10000;

% Calculate modulation index

m = Am / Ac;

fprintf('Modulation Index = %.2f\\n', m);

fprintf('Modulation Percentage = %.2f percent\\n', m*100);

% Time vector

t = 0:1/fs:2/fm;

% Generate signals

message = Am*cos(2*pi*fm*t);

carrier = Ac*cos(2*pi*fc*t);

am_signal = Ac*(1 + m*cos(2*pi*fm*t)).*cos(2*pi*fc*t);

% Plot baseband signal

figure;

subplot(4,1,1);

plot(t, message, 'b');

title('Baseband Signal');

xlabel('Time (seconds)');

ylabel('Amplitude');

grid on;

% Plot carrier signal

subplot(4,1,2);

plot(t, carrier, 'k');

title('Carrier Signal');

xlabel('Time (seconds)');

ylabel('Amplitude');

grid on;

% Plot AM signal

subplot(4,1,3);

plot(t, am_signal, 'r');

hold on;

env = Ac*(1 + m*cos(2*pi*fm*t));

plot(t, env, 'b--');

plot(t, -env, 'b--');

title('Amplitude Modulated Signal');

xlabel('Time (seconds)');

ylabel('Amplitude');

grid on;

% Plot demodulated message component

subplot(4,1,4);

plot(t, env - Ac, 'r');

title('Demodulated Output');

xlabel('Time (seconds)');

ylabel('Amplitude');

grid on;

5. MATLAB OUTPUT

The following output shows the baseband signal, carrier signal, amplitude-modulated signal with its envelope, and the demodulated message component.

6. RESULT

The modulation index was calculated in MATLAB as m = Am/Ac = 2/5 = 0.4, corresponding to 40% modulation. The plotted AM waveform and its envelope were obtained successfully. Since the modulation index is less than 1, the signal is under-modulated.