EXPERIMENT – 7

CALCULATION AND VERIFICATION OF FREQUENCY DEVIATION AND MODULATION INDEX OF FM SIGNALS USING MATLAB SIMULATION

AIM

To calculate and verify the frequency deviation and modulation index of a Frequency Modulated (FM) signal using MATLAB simulation and to observe the message signal and FM signal.

SOFTWARE REQUIRED

• MATLAB (MATLAB R2020a or later recommended)

PARAMETERS USED

| Parameter | Value |
| --- | --- |
| Message amplitude (Am) | 2 V |
| Message frequency (fm) | 500 Hz |
| Carrier amplitude (Ac) | 1 V |
| Carrier frequency (fc) | 10000 Hz |
| Frequency sensitivity (kf) | 250 Hz/V |
| Sampling frequency (fs) | 100000 Hz |
| Simulation duration (T) | 0.01 s |

FORMULAE USED

Frequency Deviation:

Δf = kf × Am

FM Modulation Index:

β = Δf / fm

CALCULATION

Given: kf = 250 Hz/V, Am = 2 V, and fm = 500 Hz.

Frequency deviation: Δf = 250 × 2 = 500 Hz.

Modulation index: β = 500 / 500 = 1.

MATLAB CODE

% Calculation and Verification of Frequency Deviation
% and Modulation Index of FM Signal
clc;
clear;
close all;

%% Message Signal Parameters
Am = 2; % Message amplitude (V)
fm = 500; % Message frequency (Hz)

%% Carrier Signal Parameters
Ac = 1; % Carrier amplitude (V)
fc = 10000; % Carrier frequency (Hz)

%% FM Parameters
kf = 250; % Frequency sensitivity (Hz/V)
fs = 100000; % Sampling frequency (Hz)
T = 0.01; % Simulation duration (s)

%% Time Vector
t = 0:1/fs:T;

%% Message Signal
m = Am*cos(2*pi*fm*t);

%% Frequency Deviation
delta_f = kf*Am;

%% Modulation Index
beta = delta_f/fm;

%% FM Signal
fm_signal = Ac*cos(2*pi*fc*t + beta*sin(2*pi*fm*t));

%% Display Parameters
fprintf('Carrier Frequency (fc) = %.0f Hz\n',fc);
fprintf('Message Frequency (fm) = %.0f Hz\n',fm);
fprintf('Message Amplitude (Am) = %.0f V\n',Am);
fprintf('Frequency Sensitivity (kf) = %.0f Hz/V\n',kf);
fprintf('\nFrequency Deviation (Delta f) = %.2f Hz\n',delta_f);
fprintf('Modulation Index (Beta) = %.2f\n',beta);

%% Verification
beta_verify = delta_f/fm;
fprintf('Verified Modulation Index = %.2f\n',beta_verify);

%% Verification Check
tolerance = 1e-6;
if abs(beta - beta_verify) < tolerance
    fprintf('\nVerification Successful\n');
else
    fprintf('\nVerification Failed\n');
end

%% Plotting
figure;
subplot(2,1,1);
plot(t,m,'LineWidth',1.5);
grid on;
title('Message Signal');
xlabel('Time (s)');
ylabel('Amplitude (V)');

subplot(2,1,2);
plot(t,fm_signal,'LineWidth',1);
grid on;
title('FM Signal');
xlabel('Time (s)');
ylabel('Amplitude (V)');

RESULT / EXECUTION OF OUTPUT

The MATLAB program was executed successfully. The calculated frequency deviation is 500 Hz and the FM modulation index is 1. The calculated modulation index was verified using β = Δf/fm, and the verification was successful.

Figure: MATLAB execution output showing Message Signal and FM Signal

OBSERVATION

The message signal is a 500 Hz cosine waveform with an amplitude of 2 V. The FM signal has a carrier frequency of 10 kHz and its instantaneous frequency varies according to the message signal. The frequency deviation is 500 Hz, resulting in an FM modulation index of 1.

CONCLUSION

Thus, the frequency deviation and modulation index of the FM signal were successfully calculated and verified using MATLAB simulation. The frequency deviation obtained is 500 Hz and the modulation index is 1. The verification was successful.